# Workshop Case Study: Vehicle Light weighting, Cost & CO₂ Reduction

## Objective

Reduce the vehicle’s curb mass by at least 200kg while:

- Meeting strength and corrosion requirements

- Minimizing cost

- Calculating fuel saved and CO₂ reduction

## Part 1 – Case Study Overview

Scope

Make material substitutions and design changes at the subsystem level.

The final design must:

- Comply with Table 3 requirements

- Achieve ≤ 1,600 kg curb mass (from current 1,800 kg)

Table 1: Baseline Vehicle

| Subsystem | Baseline Materials | Mass (kg) |
| --- | --- | --- |
| Body & Chassis | Mild Steel | 585 |  |
| Exterior Panels | Mild Steel | 490 |  |
| Powertrain (Engine + Trans) | Cast iron (60%) + Steel (40%) | 410 |  |
| Suspension & Breaks | Steel | 155 |  |
| Wheels | Steel | 70 |  |
| Other (Exhaust, Tank...) | Steel | 90 |  |
| Total: |  | 1800 |  |

Use Table 2 and Asbhy diagram to choose material which suits.

Table 2: Material Option Cards

| Material | Cost rate per kg | Corrosion Rating (1–5) | Notes / Weight Factor* |
| --- | --- | --- | --- |
| Mild Steel | 1 | 4 | *1.00 |
| HHS | 1.5 | 4 | *0.81 |
| Aluminum Alloy | 4 | 3 | *0.55 |
| Magnesium Alloy | 6 | 2 | *0.5 |
| CFRP (Carbon Fiber) | 20 | 5 | *0.40 |
| Titanium Alloy | 30 | 5 | *0.55 |

Note: Print ashby diagram seperately.

Table 3: Requirements of Parts

| Subsystem | Min Yield Strength (MPa) | Corrosion Environment |
| --- | --- | --- |
| Body & Chassis | 350 | Medium |  |
| Exterior Panels | 150 | High |  |
| Powertrain (Engine + Trans) | 200 | Medium (oily) |  |
| Suspension Arms | 500 | High |  |
| Brakes (Caliper/Disk) | 400 | High + Heat |  |
| Wheels | 300 | High |  |
| Interior & Seats | 80 | Low |  |
| Exhaust Zone | 200 | High |  |
| Fasteners | 600 | High |  |

Formulas

- New Mass

mnew=mold×Weight Factor

- Cost Change

∆Cost=mnew× Costnew-(mold× Costold)

## Part 2 – Fuel &  CO2 Reduction

- To calculate total fuel saved by reducing the mass use the following formula:

Fuel Saved (L) = Mass Impact Factor (MIF) × Mass Reduction (kg) × Total Lifetime Distance (km)​

MIF=0.28L100km/100kg=0.000028

Total Lifetime Distance=250000 km

- To calculate saved greenhouse gas emissions use following formula

GHG Emissions Saved = Fuel Saved (liters) × Emission Factor

Emission Factor: 2.82 kg × CO2-eqliter