# Workshop Activity: Vehicle Light weighting & Cost Analysis

## Objective

In this activity, you will:

- Calculate the baseline weight of each vehicle subsystem.

- Determine the mass of each material in the baseline vehicle.

- Redesign the three heaviest subsystems (Body Non-Structural, Body Structural, and Powertrain) by replacing steel with a lighter alternative material (assigned to your group).

- Calculate the weight reduction and the cost of your redesigned subsystems.

- Compare your results with other groups.

## Background

The baseline vehicle is made primarily from mild steel, cast iron, aluminum, copper, and other common automotive materials.

Light weighting reduces vehicle mass and can improve performance, fuel efficiency, and environmental impact.

## Given Data

Table 1 – Baseline Vehicle Weight Distribution

| Subsystem | Mass Fraction (%) | Baseline Primary Material |
| --- | --- | --- |
| Body Non-structural | 20 | Mild Steel (Note: ignored glass and interior) |  |
| Body Structure | 18 | Mild Steel |  |
| Front Suspension | 5.3 | Steel |  |
| Rear Suspension | 3.9 | Steel |  |
| Braking | 3.6 | Cast iron |  |
| Powertrain | 23 | Cast Iron (engine block) %60 + Mild Steel (transmission parts) %40 |  |
| Fuel & Exhaust | 3.5 | Steel |  |
| Steering | 1.3 | Steel |  |
| Tires & Wheels | 6.5 | Steel |  |
| Electrical | 4.5 | Copper |  |
| Cooling | 2.2 | Aluminum |  |
| Bumpers | 2 | Steel |  |
| Closures | 6.2 | Mild Steel |  |
| Total Weight: | 100 | 2000kg |  |

Table 2 – Weight Reduction Coefficients

| Weight Reduction Coefficients Table |  |  |  |
| --- | --- | --- | --- |
| Material | Mass Reduction Range | Middle Reduction % | Weight Coefficient |  |
| Mild Steel | 0% | 0 | 1 |  |
| High Strength Steel (HSS) | 10–28% | 19% | 0.81 |  |
| Advanced High Strength Steel (AHSS) | 15–25% | 20% | 0.8 |  |
| Aluminum & Al Matrix Composites | 30–60% | 45% | 0.55 |  |
| Carbon Fiber Composites | 50–70% | 60% | 0.4 |  |
| Magnesium | 30–70% | 50% | 0.5 |  |

Table 3 – Material Prices

| Material Prices |  |
| --- | --- |
| Material | 2025 ($/kg) |  |
| Mild | 0.83 |  |
| HSS | 1.08 |  |
| AHSS | 1.15 |  |
| Al | 2.61 |  |
| Cast Iron | 0.40 |  |
| Mag | 2.33 |  |
| CFRP | 34.5 |  |